# ELEC 3EY4 – Lab 2

# Notes

- Shift print screen = ss

# Objective 1 - Introduction to Robot Operation System

## Notes

What is ROS?

- ROS is an open-source, BSD-licensed framework for building and controlling robotic systems.

- It uses a publish/subscribe messaging model to allow independent nodes (programs) to communicate.

ROS Architecture:

- Nodes: Independent executables that perform specific tasks (e.g., sensor drivers, controllers).

- Topics: Named channels over which nodes exchange messages.

- Master: A central service that helps nodes discover each other and establish communication.

Communication Models:

- Publisher-Subscriber: Asynchronous, one-way data flow.

- Publisher: Sends messages to a topic.

- Subscriber: Receives messages from a topic.

- Services: Synchronous request-response communication between nodes.

- Client: Sends a request.

- Server: Processes the request and returns a response.

ROS Terminology:

- Workspace: Directory where ROS packages are stored and built.

- Package: A collection of ROS nodes, libraries, messages, and dependencies.

- Messages: Data structures exchanged between nodes.

Common ROS Commands:

- roscore: Starts the ROS Master.

- rostopic: Interacts with topics (list, echo, pub, info).

- rosnode: Interacts with nodes (list, info).

- rosmsg: Displays information about message types.

## Practice Info

https://wiki.ros.org/Nodes

rosnode - ROS Wiki

Topics - ROS Wiki

rostopic - ROS Wiki

Services - ROS Wiki

rosservice - ROS Wiki

# Objective 2 - Connecting to Jetson Nano

## Notes

Default username: caruser

Default password: ELECENG 3EY4

Power the Jetson Nano with the portable switched-mode power supply in your AEV kit. Plug the power supply into the round barrel jack on the Jetson Nano module (see Figure 7). At this point, connect Jetson Nano to a monitor via an HDMI cable. You should also insert a wired mouse and a keyboard into USB ports (unplug VESC if not enough ports)

# Objective 3 - ROS Basics Practical Example

## Notes

- Ctrl + Alt + T - open new Terminal Window

- roscore - starts the ROS Master

- rostopic - gives list of all the options that the rostopic command can do

- rostopic list – lists all published topics

- rostopic pub /topicName std_msgs/dataType "dataContent" - publishes the message, sending it to the topic called TopicName, using the message format std_msgs of data type dataType with the data field of “dataContent”

- rostopic echo /topicName – displays messages published to topicName

  - Ctrl + C – to stop echoing

- rosnode list – lists all active nodes

- rosnode info /nodeName - displays info about nodeName

  - including publications, subscriptions, services, connections, etc.

- rostopic info /topicName - displays info about topicName

## Question 1

What is the purpose of the entries /rosout and /rosout_agg listed in Figure 10? Explain in detail.

/rosout:

- Receives raw log messages from individual nodes.

- Used for ROS logging and diagnostics.

- When ROS nodes publish log messages (e.g. using ROS_INFO, ROS_WARN,  ROS_ERROR), those messages are sent to the /rosout topic.

- The rosout node (started automatically with roscore) subscribes to /rosout and aggregates these log messages, which can then be viewed using tools like rqt_console or rosconsole.

- Monitors the system's status and debug issues.

- All nodes, regardless of location will appear on this topic

/rosout_agg:

- Provides cleaned and aggregated log stream for easier system monitoring.

- An aggregated version of /rosout.

- Collects log messages from multiple nodes across the ROS network into a single stream.

- Unlike /rosout, which may receive duplicate or fragmented logs from different sources, /rosout_agg provides a consolidated, de-duplicated feed of system-wide log messages.

- Useful for centralized logging and monitoring in distributed robotic systems

## Question 2

Break down the parts of the command in Figure 11 and explain each function: rostopic pub, /hello, std_msgs/String, “Hello Robot”.

Command used: rostopic pub /hello std_msgs/String “Hello Robot”

rostopic pub

- rostopic - ROS command-line tool for interacting with topics

- pub - subcommand that stands for "publish"

- rostopic pub - manually publish a message to a specific ROS topic

/hello

- topic's name to which the message will be published

- topic is called /hello

std_msgs/String

- specifies the message type being published (message type here is String)

- tells ROS the structure and format of the message

- std_msgs - standard ROS message package

  - Basic data types (String, Int32, Float64, Bool, etc.)

- String - message type within that package, which contains a single string field called data

"Hello Robot"

- the actual data being sent in the message

- The data that populates the corresponding data field of the message

Overall, the command instructs ROS to:

- Use the rostopic tool to publish a message.

- Send it to the topic named /hello.

- Use the std_msgs/String message format.

- With the content "Hello Robot".

## Question 3

Write and explain in your own words a summary of what you did in this objective and what you learned from it. Include screenshots of the output of the rosnode info /rostopic/_…. command and of the rostopic info /hello command.

- Ctrl + Alt + T - open new Terminal Window

- roscore - starts the ROS Master

- rostopic - gives list of all the options that the rostopic command can do

- rostopic list – lists all published topics

- rostopic pub /topicName std_msgs/dataType "dataContent" - publishes the message, sending it to the topic called TopicName, using the message format std_msgs of data type dataType with the data field of “dataContent”

- rostopic echo /topicName – displays messages published to topicName

  - Ctrl + C – to stop echoing

- rosnode list – lists all active nodes

- rosnode info /nodeName - displays info about nodeName

  - including publications, subscriptions, services, connections, etc.

- rostopic info /topicName - displays info about topicName

# Objective 4 - Setting up udev Rules

## Notes

## Question 4

Explain why you need to apply the following commands and what they mean.

NOTE: If you change a udev rule for any reason in the future, you must execute these commands after making your changes and saving the files.

$ sudo udevadm control --reload-rules 
$ sudo udevadm trigger

sudo udevadm control --reload-rules

- reloads the udev rules from the /etc/udev/rules.d/ directory without requiring a system reboot.

- the changes are not automatically picked up by the udev system. This command tells udev to reread all rule files and update its internal rule database.

sudo udevadm trigger

- forces udev to re-process all connected devices and apply the newly loaded rules.

- Even after reloading the rules, existing devices are not automatically reassessed. This command triggers udev to re-evaluate all devices and apply the appropriate rules (e.g., setting permissions, creating symlinks) based on the updated configurations.

Why these commands are necessary together:

- Reloading rules updates what udev knows.

- Triggering updates what udev does with connected hardware.

Without these commands:

- New or edited rules would only take effect after a system reboot.

- Devices might keep old permissions or not be detected with the new symlinks.

# Objective 5

## Question 5

Put an object such as a notebook or a cardboard at 270 degree around the lidar (at least 20 cm away), move the object back and forth and explain how and why the numbers change. Add screenshots of the array ranges when the object is the closest to the car and when it is the furthest from the car.

- We observe the distance the LIDAR detects to the closest object by specifying a parameter of 270 degrees in the command rostopic echo scan/ranges[270]

- The data seen in the terminal window is the distance measurement (in meters) obtained by the LIDAR at that specified angle. We can see in the photo on the left where the notebook is small, the data being reported showcases that the LIDAR is in close proximity with the notebook (between 15.9 to 18.9 cm)

- The data on the right confirms that the distance measurement increases as we move the notebook further away from the LIDAR sensor (around 38 to 39 cm)

| Array ranges when notebook is close to lidar | Array ranges when notebook is far from lidar |
| --- | --- |

## Question 6

Take a screenshot of the rviz page showing the point cloud map of the surrounding of the lidar and explain your observations of what the screenshot shows. Validate your observations by moving objects around the lidar and observe the change in the point cloud map.

- The rviz page confirms that the LIDAR is in working order by displaying both adjacent walls in the point cloud map as seen below.

- We can verify the accuracy of the point cloud map by moving a wallet around and seeing the position being replicated in the rviz window.

The protruding edge on the far right of the screenshot corresponds to the windowsill of the lab room, further verifying the accuracy of the LIDAR

# Objective 6

## Question 7

In your report, include a screenshot of the output with the 8-bit unsigned integer range from 0 to 255 as you echo /camera/color/image_raw and /camera/depth/image_rect_raw topics.

Integer range of /camera/color/image_raw Integer range of /camera/depth/image_rect_raw

## Question 8

Take a screenshot of the camera and depth camera simultaneously in one screenshot (Hint: you can do that by adding another camera page and selecting the topic you need). Explain briefly how the depth camera represents the raw coloured image. Also, take a screenshot of the raw coloured image showing ALL your team members.

- The depth camera represents the raw colored image by having objects closer to the camera be much darker, and then have objects that are at a further distance become brighter as you move far away from the camera itself

- We can see in the photo below the team member closest to camera is essentially black in color, with the other two team members depicted with a greyer tone as they are further away. The far back wall of the lab room is depicted with a light grey color identifying that the depth of that object is large in relation to the darkly lit team members

# Objective 7

## Question 9

The above explanation should have given you an idea about the parameters you added to the launch file. In your report, explain each line that you typed into the launch file (e.g., the four lines in the middle).

- The <node name> line specifies a ROS node with the name of “joy_node” that will be published once the roslaunch command is ran. The node is launched using the type “joy_node” from the “joy” package. The type attribute must have a corresponding executable with the same name.

- The <param> tag defines parameters that are to be stored on a network and will be retrieved at runtime. In this case, as the <param> tag is used within a <node> tag, those parameters are only accessible to that specific node

- For the first line with name = “dev”, the value of the parameter is set to js0 which is the controller itself

- The parameter with the name “autorepeat_name” defines that at every second, 2 new messages will be published if nothing is moved/pressed on the controller

- The “coalesce_interval” parameter sets the rate at which the messages are published

## Question 10

[A, B, X, Y, Back, Start, Logitech (home), LB, RB, L3, R3]

| Buttons | Pictures |
| --- | --- |
| A |  |
| B |  |
| X |  |
| Y |  |
| Back |  |
| Start |  |
| Logitech (home) |  |
| LB |  |
| RB |  |
| L3 |  |
| R3 |  |

[Arrow L/R, Arrow U/D, LT, RJ L/R, RJ U/D, RT, LJ L/R, LJ U/D]

| Axis | Picture |
| --- | --- |
| Arrows: L/R Left is +ve Right is -ve |  |
| Arrows: Up/Down Up is +ve Down is -ve |  |
| Left Trigger unpressed=1 pressed=-1 |  |
| Right joystick: L/R Left is +ve Right is -ve |  |
| Right Joystick: Up/down Up is +ve Down is -ve |  |
| Right Trigger unpressed=1 pressed=-1 |  |
| Left Joystick: Left/Right Left is +ve Right is -ve |  |
| Left Joystick: Up/Down Up is +ve Down is -ve |  |