# ELEC 3EY4 – Lab 2

# Notes – overall notes from each objective, instead of each objective having its own notes?

# Objective 1 - Introduction to Robot Operation System

## Notes

What is ROS?

- ROS is an open-source, BSD-licensed framework for building and controlling robotic systems.

- It uses a publish/subscribe messaging model to allow independent nodes (programs) to communicate.

ROS Architecture:

- Nodes: Independent executables that perform specific tasks (e.g., sensor drivers, controllers).

- Topics: Named channels over which nodes exchange messages.

- Master: A central service that helps nodes discover each other and establish communication.

Communication Models:

- Publisher-Subscriber: Asynchronous, one-way data flow.

- Publisher: Sends messages to a topic.

- Subscriber: Receives messages from a topic.

- Services: Synchronous request-response communication between nodes.

- Client: Sends a request.

- Server: Processes the request and returns a response.

ROS Terminology:

- Workspace: Directory where ROS packages are stored and built.

- Package: A collection of ROS nodes, libraries, messages, and dependencies.

- Messages: Data structures exchanged between nodes.

Common ROS Commands:

- roscore: Starts the ROS Master.

- rostopic: Interacts with topics (list, echo, pub, info).

- rosnode: Interacts with nodes (list, info).

- rosmsg: Displays information about message types.

## Practice Info

https://wiki.ros.org/Nodes

rosnode - ROS Wiki

Topics - ROS Wiki

rostopic - ROS Wiki

Services - ROS Wiki

rosservice - ROS Wiki

# Objective 2 - Connecting to Jetson Nano

## Notes

Default username: caruser

Default password: ELECENG 3EY4

Power the Jetson Nano with the portable switched-mode power supply in your AEV kit. Plug the power supply into the round barrel jack on the Jetson Nano module (see Figure 7). At this point, connect Jetson Nano to a monitor via an HDMI cable. You should also insert a wired mouse and a keyboard into USB ports (unplug VESC if not enough ports)

# Objective 3 - ROS Basics Practical Example

## Notes

- Ctrl + Alt + T - open new Terminal Window

- roscore - starts the ROS Master

- rostopic - gives list of all the options that the rostopic command can do

- rostopic list – lists all published topics

- rostopic pub /topicName std_msgs/dataType "dataContent" - publishes the message, sending it to the topic called TopicName, using the message format std_msgs of data type dataType with the data field of “dataContent”

- rostopic echo /topicName – displays messages published to topicName

  - Ctrl + C – to stop echoing

- rosnode list – lists all active nodes

- rosnode info /nodeName - displays info about nodeName

  - including publications, subscriptions, services, connections, etc.

- rostopic info /topicName - displays info about topicName

## Question 1

What is the purpose of the entries /rosout and /rosout_agg listed in Figure 10? Explain in detail.

/rosout:

- Receives raw log messages from individual nodes.

- Used for ROS logging and diagnostics.

- When ROS nodes publish log messages (e.g. using ROS_INFO, ROS_WARN,  ROS_ERROR), those messages are sent to the /rosout topic.

- The rosout node (started automatically with roscore) subscribes to /rosout and aggregates these log messages, which can then be viewed using tools like rqt_console or rosconsole.

- Monitors the system's status and debug issues.

/rosout_agg:

- Provides cleaned and aggregated log stream for easier system monitoring.

- An aggregated version of /rosout.

- Collects log messages from multiple nodes across the ROS network into a single stream.

- Unlike /rosout, which may receive duplicate or fragmented logs from different sources, /rosout_agg provides a consolidated, de-duplicated feed of system-wide log messages.

- Useful for centralized logging and monitoring in distributed robotic systems

## Question 2

Break down the parts of the command in Figure 11 and explain each function: rostopic pub, /hello, std_msgs/String, “Hello Robot”.

Command used: rostopic pub /hello std_msgs/String “Hello Robot”

rostopic pub

- rostopic - ROS command-line tool for interacting with topics

- pub - subcommand that stands for "publish"

- rostopic pub - manually publish a message to a specific ROS topic

/hello

- topic's name to which the message will be published

- topic is called /hello

std_msgs/String

- specifies the message type being published

- tells ROS the structure and format of the message

- std_msgs - standard ROS message package

  - Basic data types (String, Int32, Float64, Bool, etc.)

- String - message type within that package, which contains a single string field called data

"Hello Robot"

- the actual data being sent in the message

- The data that populates the corresponding data field of the message

Overall, the command instructs ROS to:

- Use the rostopic tool to publish a message.

- Send it to the topic named /hello.

- Use the std_msgs/String message format.

- With the content "Hello Robot".

## Question 3

Write and explain in your own words a summary of what you did in this objective and what you learned from it. Include screenshots of the output of the rosnode info /rostopic/_…. command and of the rostopic info /hello command.

*refer to this objective’s notes section*

***INCLUDE SCREENSHOTS***

# Objective 4 - Setting up udev Rules

## Notes

## Question 4

Explain why you need to apply the following commands and what they mean.

NOTE: If you change a udev rule for any reason in the future, you must execute these commands after making your changes and saving the files.

$ sudo udevadm control --reload-rules 
$ sudo udevadm trigger

1. sudo udevadm control --reload-rules

This command reloads the udev rules from the /etc/udev/rules.d/ directory without requiring a system reboot.

Why it’s needed:
When you edit a udev rule file, the changes are not automatically picked up by the udev system. This command tells udev to reread all rule files and update its internal rule database.

2. sudo udevadm trigger

This command forces udev to re-process all connected devices and apply the newly loaded rules.

Why it’s needed:
Even after reloading the rules, existing devices are not automatically reassessed. This command triggers udev to re-evaluate all devices and apply the appropriate rules (e.g., setting permissions, creating symlinks) based on the updated configurations.

Why these commands are necessary together:

Reloading rules updates what udev knows.

Triggering updates what udev does with connected hardware.

Without these commands:

New or edited rules would only take effect after a system reboot.

Devices might keep old permissions or not be detected with the new symlinks.

Example Scenario:

You add a rule for the LIDAR (rplidar.rules) so it appears as /dev/sensors/rplidar. After saving the file:

Reload rules → udev now knows about the new rule.

Trigger → udev applies the rule to the currently connected LIDAR, creates the symlink, and sets permissions to 0777.

# Objective 5

## Question 5

## Question 6

# Objective 6

## Question 7

## Question 8

# Objective 7

## Question 9

## Question 10