# Lab 7: Autonomous Driving with MacAEV

ELECENG 3EY4 – Electrical Systems Integration Project

Amaiam Haque – haquea24 – 400520641

Aayan Siddiqui – sidda2 – 400386976

Jasmine Smith – smitj19 – 400262687

As a future members of the engineering profession, the student is responsible for performing the required work in an honest manner, without plagiarism and cheating. Submitting this work with my name and student number is a statement and understanding that this work is my own and adheres to the Academic Integrity Policy of McMaster University and the Code of Conduct of the Professional Engineers of Ontario.

# Activity 1

Determine the steady-state tracking error in response to a constant command in the closed-loop control. Explain how, in theory, the parameters kp and kd would affect the closed-loop control system response.

From our knowledge in ELECENG 3CL4, we know that certain type numbers of system correspond to different values of steady-state error. As we are given the closed loop transfer function, we can derive the open loop transfer function and recognize what type it is, and compare that to what the steady state error is for that type for a unit step function (as the lab manuals refers to a constant command).

Given the closed loop transfer function Hcls=kps2+kds+kp,

The general form of a closed loop transfer function is Hcls=Gs1 + Gs (for unity feedback which is assumed in this case).

Gs1 + Gs=kps2+kds+kp

Gss2+kds+kp=kp1 + Gs

Gss2+Gskds+Gskp=kp+ Gskp

Gss2+Gskds=kp

Gss2+kds=kp

Gs=kpss+kd

As we can see, since there is only one 1/s term in the open loop transfer function, that this is a type 1 system, and that its steady state error is 0 for a unit step input. In terms of how kd and kp affect the closed loop system response, we can compare Hcl(s) to the standard order form for a second order system,

Hs=ωn2s2+2ζωns+ωn2

ωn2=kp

ωn=kp

We can see that Kp increases the natural frequency of the system, improving our response time, but adding more oscillations/higher overshoot.

For the second term,

2ζωn=kd

ζ=kd2ωn,   ωn=kp

ζ=kd2kp

We can see that a higher kd means that the damping ration increases, which will reduce the percent overshoot seen and the oscillations, but slow the response. We can also see that increasing kp will have the opposite effect, and confirms what was mentioned earlier (faster response, more oscillations and higher overshoot).

# Activity 2

Explain what velocity control strategy in (19) tries to accomplish.

The main goal of this velocity control command is to adjust the speed of the bot when an object appears in front of the bot. This is done through the use of an exponential function to ensure that the bot stops in a smooth manner instead of jerking/instantly coming to a halt putting stress on the components itself. It uses the distance of the closest object in front of it (dob) and compares it to a safe stopping distance (dstop) to adjust the exponential term (if dob – dstop is large as the object is far away, the vehicles velocity will be near it’s max, and while it’s small, the speed decreases accordingly). The dt parameter within the power of the exponential can control the rate of decay for the velocity overall, where when the parameter is small the braking is aggressive, and is more gradual when the parameter is large.

# Activity 4

Observe the vehicle behavior in the simulation environment as it drives along the hallway. Where do you see the algorithm struggling and why? Explore and explain the impact of the controller gaining kp and kd on the response characteristics. How would the mechanical rotation frequency of the LiDAR impact the selection of these controller gains?

Due to the nature of the algorithm (detecting the walls through two fixed beams on either side of the bot), the algorithm struggles heavy around corners as it can’t accurately place a wall due to the LIDAR bams not returning accurate measurements of walls at certain angles (if the wall is not straight and perfectly aligned with the bot, it can’t accurately build a virtual wall for the bot to stay in between of). In the middle of a straight hallway however, the algorithm can perfectly center the bot without any oscillations seen in rviz (no overcorrection within the simulation, this was not the case when running the code on the bot which we will discuss later).

If we were into increase kp, the bot would try to return to the center of the wall much faster, but will have more oscillations and a higher percent overshoot (which would then cause the bot to overcorrect after, creating a sort of “s” pattern due to having to overcorrect each time). We could then increase kd to reduce the oscillations seen and having a smoother response because of it, but could also slow the response so that the bot doesn’t center itself quick enough.

The mechanical frequency of the LIDAR impacts how high/low we should set our kp and kd values. If our mechanical frequency of LIDAR is too slow, there’s more delay between measurements of the walls, meaning that if the bot is off center, by the time the bot tries to correct that error, the bot itself is even more off center. This means we need a lower kp so the corrections are less aggressive and the bot doesn’t become too unstable. The kd value needs to be increased in order to reduce the oscillations seen due to that delay in error. If we increase the mechanical frequency of the LIDAR, the updates between scans are much more frequent meaning we can have a much more aggressive correction of the bot through kp, and don’t have to rely on kd for reducing oscillations.

# Activity 5

- Do you observe any difference in the vehicle behavior between the simulations and the experiment? How do you explain these potential differences?

In the real-world experiment, the AEV robot had a tendency to oscillate significantly more. It was more sensitive to the wall distances and struggled to reach steady-state. Some modifications to the wall-following algorithm were implemented to account for this issue.

This difference was likely due to noisy scans from the LiDAR. Given the angles chosen for the wall scans, there was a significant distance between the first scan of a wall and the second scan of a wall. The greater the distance between the two angles, the greater the room for error and the impact of noise. Additionally, the distance between the angles resulted in the AEV struggling far more with corners compared to the simulation.  Friction may have also had an impact on the AEV’s steering and turning response.

- The RPLiDAR A2M8 scanner has two important parameters, the number of samples/second and mechanical scan frequency. You can refer to the RLiDAR specifications here to find out about the values of these parameters and their range. Discuss the trade-offs involved affecting the selection of these parameters in the context of self-driving. For example, with a fixed signal sampling rate, what happens if you increase or decrease mechanical rotation frequency? What is the impact of increasing or decreasing sampling rate with a fixed mechanical rotation frequency?

  - Fixed sampling rate, changing mechanical rotation frequency:

Increasing the rotational frequency would increase the rate of complete visual updates. Each complete rotation provides a “picture” of the AEV’s full surroundings. This complete picture update would occur more frequently, allowing for more frequent updates to the AEV’s surroundings. Additionally, the system would be making fewer calculations per rotation, likely resulting in a faster response time. The trade-off would be in the resolution. A faster rotation would result in fewer scans per rotation and thus a lower resolution of the AEV’s surroundings. A lower resolution means that the AEV  is less likely to respond correctly to its environment and may miss obstacles.

  - Fixed rotation frequency, changing sampling rate

The sampling rate has a similar set of tradeoffs. Increasing the sampling rate would increase the amount of samples per revolution. This would increase the resolution of the AEV’s surroundings and allow to respond to its in environment in great detail (i.e. less likely to miss obstacles). It may also increase the time to respond to its environment as it will have more data points per revolution to calculate and process.

# Activity 7

Based on your observations, what are the key parameters of the algorithm that affect its performance in your simulations? What are the trade-offs in selecting the values of these parameters?

The following parameters impacted the simulation performance:

K_p: The proportional constant impacts error correction/oscillation response. Increasing Kp results in higher and more responsive correction. This however could result in higher oscillations, jerky movement, and a higher overshoot. Decreasing Kp increases the stability of the system, but slows down error correct and results in a higher steady state error.

K_d: The derivative constant impacts reaction time and noise sensitivity. Increasing Kd improves dampening allowing for steady state to be reached faster. Overshoot and rise time both decrease. It also increases sensitivity to noise. Decreasing Kd results in a faster reaction/correction to new inputs. Thus increases the AEV’s overshoot, and increases the oscillatory behavior.

Vehicle_velocity: The vehicle velocity impacts stability, navigation abilities and reaction time. Increasing vehicle velocity limits the amount of the environment that the AEV can view before driving through it. This reduces the AEV’s ability to react to its environment. Decreasing the vehicle velocity improves the AEV’s ability to observe and navigate through its environment.

safe_distance: Increasing safe_distance results in a higher likelihood of avoiding obstacles, and an earlier detection/reaction to obstacles. However, it could also decreas the amount of valid gaps. Decreasing safe_distance increases risk of collision in obstacles and decreases reaction time. It also allows the AEV to driver in tighter gaps that it can still fit through.

Stop_distance: Increasing stop distance improves collision avoidance. However, it also results in more frequent stopping and could limit the amount of gaps the AEV can drive through.

right_beam_angle: Increasing right_beam_angle increases the bias towards right side gap detection. Decreasing right_beam_angle increases bias towards the front and improves gap detection in the center/front.

left_beam_angle: Increasing left _beam_angle increases the bias towards left side gap detection. Decreasing left _beam_angle increases bias towards the front and improves gap detection in the center/front.

Heading_beam_angle: Increasing heading beam angle allows for a wide visibility of the AEV’s environment and allowing it to see more obstacles in its peripheral. However, it also increases computation time which decreases response time. Decreasing heading beam angle allows the AEV to focus on the obstacles in front of it. It will have a faster response time, but may not notice obstacles on the side.

Stop_distance_decay: Increasing stop_ distance _decay allows for smoother deacceleration.

# Activity 8

Briefly report on your observations from the experiments.

In self-driving mode, the AEV performed very well in straight, empty hallways. It stayed within the center of the hallway with minimal overshoot and little to no oscillatory behavior. It promptly found the center of the hallways regardless of its initial placement.

The AEV consistently successfully avoided all obstacles and navigated the hallway containing both corners and multiple obstacles. When approaching corners and obstacles, the AEV had a tendency to take wide turns, but would occasionally respond in close proximity to the obstacle. This shorter response time is likely due to the wide turns. When the AEV made wide turns, it often turned towards a new obstacle, forcing the AEV to turn again. The AEV slowed down as it approached obstacles, which allowed it to have a decreased response distance, and allowed it to take wider turns. The AEV also consistently found the largest and most convenient gap between obstacles.